## RC CAR TINKER CAD STIMULATION AND ARDUINO PROGRAMMING

### Project Overview

This project demonstrates the design, simulation, and programming of a basic RC Car control system using Arduino Uno in  Tinkercad. The RC car movement is controlled using push buttons (simulating a remote controller), and a motor driver IC (L293D) is used to drive a DC motor safely.

### 1. Objectives

- Design and simulate an RC car control circuit in Tinkercad.

- Understand motor direction control using an H-Bridge (L293D).

- Program Arduino to control motor movement (forward, backward, stop).

- Analyze the working of an RC car control logic.

### 2. Introduction

An RC (Remote Controlled) car is a vehicle controlled wirelessly using a controller. In this Tinkercad-based project, physical RF modules are replaced by push buttons to simulate remote control signals. The Arduino processes the input commands and controls the DC motor through a motor driver IC, enabling forward and reverse movement of the car.

### 3. Block Diagram (Description)

Push Buttons → Arduino Uno → Motor Driver (L293D) → DC Motor → Wheel Movement

### 4. Components Used

| Component | Purpose |
| --- | --- |
| Arduino Uno | Main controller to process inputs |
| L293D Motor Driver IC | Drives DC motor and controls direction |
| DC Motor | Provides mechanical movement |
| Push Buttons (2) | Simulate remote control inputs |
| Breadboard | Circuit prototyping |
| Jumper Wires | Electrical connections |
| Power Supply (USB / Battery) | Powers the circuit |

### 5. Wiring & Pin Configuration

| Signal / Module | Arduino Pin |
| --- | --- |
| Motor Driver IN1 | D8 |
| Motor Driver IN2 | D9 |
| Motor Driver Enable (EN1) | D10 |
| Forward Button | D2 |
| Backward Button | D3 |
| Motor Driver VCC1 | 5V |
| Motor Driver GND | GND |

Note: All grounds must be common.

### 6. Building the Circuit in Tinkercad

- Open Tinkercad → Circuits → Create New Circuit.

- Place Arduino Uno, breadboard, L293D IC, DC motor, and push buttons.

- Connect Arduino digital pins to L293D input pins.

- Connect the DC motor to L293D output pins.

- Wire push buttons to Arduino input pins.

- Ensure proper 5V and GND connections.

### 7. Arduino Code

int motorPin1 = 8;

int motorPin2 = 9;

int enablePin = 10;

int buttonForward = 2;

int buttonBackward = 3;

void setup() {

pinMode(motorPin1, OUTPUT);

pinMode(motorPin2, OUTPUT);

pinMode(enablePin, OUTPUT);

pinMode(buttonForward, INPUT);

pinMode(buttonBackward, INPUT);

digitalWrite(enablePin, HIGH);

}

void loop() {

if (digitalRead(buttonForward) == HIGH) {

digitalWrite(motorPin1, HIGH);

digitalWrite(motorPin2, LOW);

}

else if (digitalRead(buttonBackward) == HIGH) {

digitalWrite(motorPin1, LOW);

digitalWrite(motorPin2, HIGH);

}

else {

digitalWrite(motorPin1, LOW);

digitalWrite(motorPin2, LOW);

}

}

### 8. Code Explanation

- Digital pins D2 and D3 read button inputs.

- Arduino sends control signals to the L293D motor driver.

- Enable pin keeps the motor driver active.

- Based on input, the motor rotates forward, backward, or stops.

### 9. Testing & Debugging in Tinkercad

- Press the forward button → motor rotates forward.

- Press the backward button → motor rotates reverse.

- Release both buttons → motor stops.

- Check wiring if motor does not respond.

### 10. Observations

The RC car simulation successfully demonstrated directional motor control using Arduino and a motor driver. The system responded accurately to button inputs with smooth motor operation.

### 11. Applications

- RC cars and toy vehicles

- Robotics projects

- Automation systems

Educational demonstrations   Educational Learning Tool
RC car projects are widely used in schools and colleges to teach basics of Arduino programming, motor control, and embedded systems.

  Robotics Training
The project helps beginners understand robot movement, direction control, and motor driver operation, forming the foundation for advanced robotics.

  Prototype for Wireless Vehicles
This simulation represents the basic working of real RC cars, which can later be upgraded with RF, Bluetooth, or Wi-Fi modules.

  Automation and Control Systems
The same control logic is used in automated guided vehicles (AGVs) and material handling robots.

  STEM and Mini Projects
Ideal for mini projects, science exhibitions, and lab experiments using Tinkercad and Arduino.

  Testing Motor Driver Circuits
Useful for testing H-bridge motor drivers (L293D/L298N) safely in simulation before real hardware implementation.

  Game and Hobby Projects
Can be extended to create toy cars, racing robots, and hobby electronics projects.

  Research and Development
Used as a basic platform for experimenting with sensors, obstacle avoidance, and autonomous navigation.

### 12. Conclusion

This project clearly explains the working principle of an RC car using Arduino and motor driver IC. The Tinkercad simulation helps understand real-world RC vehicle control systems in a simple and effective manner.

https://www.tinkercad.com/things/di2LgWi5W2o-surprising-wolt?sharecode=Z9GjVJQomOB0hKCRbkjEBYu-6YWYkpBnJ4JDwEEOC8U